# UI Design: Operator Dashboard — fs_autonomy

Formula Student Driverless — Simulation-First Autonomy Platform (software only)

Rishi Satish  ·  Harshvardhan Singh  ·  Ayon Majumdar

Dept. of Electronics and Computer Engineering, K. J. Somaiya School of Engineering
Guide: Dr. Makarand Govind Kulkarni

# 1. What Counts as the "User Interface" Here

This project has no custom app frontend (no React/Qt/web client) — the system is a set of ROS 2 backend nodes. The actual human-facing interface the project ships is the live operator dashboard, built on Foxglove Studio (an off-the-shelf ROS visualization client) and fed by this project's own code: cone_viz.py, which republishes raw simulator cone data as a format Foxglove can render (visualization_msgs/MarkerArray), plus the real topics the mission controllers publish (/cmd, /odometry/filtered, /sim/ros_can/state_str). This is the same dashboard referenced in the earlier PoC and integration submissions when it was mentioned as “running on Foxglove.”

A Foxglove Studio layout is a JSON file — panel types, topic bindings, and screen arrangement, all human-editable — making it the direct software-project equivalent of an editable PCB layout: it is the actual design source for the screens, and it can be re-opened and edited in the same tool that renders it.

# 2. Main Screens and Navigation

The dashboard is organised as three tabs (Foxglove’s Tab panel), giving the operator a linear navigation flow matching an actual run: confirm state before driving, watch the car drive, then check estimator health after. Screens below are layout mockups of the design (not live captures — no run was active while building this submission); the panel bindings they illustrate are exactly what foxglove_layout.json defines.

Figure 1. Screen 1 — Pre-Drive: confirm mission + AS state, watch node startup log before trusting the view.

Figure 2. Screen 2 — Drive View: 3D cone/car view, live speed tracking, and the raw /cmd output — the same control loop traced in the integration submission.

Figure 3. Screen 3 — Diagnostics: EKF vs ground-truth overlay and lap counter, for post-run review.

Navigation between the three is a single tab-click, by design: an operator at a live run should never need more than one click to get from “is it safe to drive” to “is it driving correctly” to “why did it do that”.

# 3. Design Explanation

Screen 1 exists because of a real incident class this project designs around. an operator must see AMIState/ASState before trusting anything else on screen — the mission-gating logic every controller runs (see the integration submission, Section 3.1) is exactly this same state, so the dashboard surfaces it first.

Screen 2's 3D view is driven by cone_viz.py, not a generic point cloud. it recolours and resizes markers per the project's own cone classification (classify(), same function used in trackdrive_driver.py), so what the operator sees matches what the controller is reasoning about — not a raw unlabeled scan.

The /cmd raw-message panel deliberately exposes drive.speed. to make the known simulator defect (drive.speed silently ignored, Defect 1 from the IEEE paper) visible on screen rather than hidden, so an operator isn't misled into thinking that field does anything.

Screen 3's EKF-vs-ground-truth plot is the same comparison as the PoC validation submission. reused here as a live/continuous view rather than a one-off offline measurement.

# 4. Frontend / Interface Source Files

| File | Role |
| --- | --- |
| foxglove_layout.json | Editable design source for all three screens — panel types, topic bindings, tab navigation. Import via Foxglove Studio → Layout → Import from file. |
| cone_viz.py | Frontend data-adapter: converts raw simulator cone messages into the classified, coloured markers Screen 2 renders. |

Excerpt of the Screen 2 / Screen 3 panel bindings from foxglove_layout.json:

"Plot!drive_speed": {
  "paths": [
    { "value": "/odometry/filtered.twist.twist.linear.x", "label": "estimated speed" },
    { "value": "/cmd.drive.acceleration", "label": "commanded accel" }
  ],
  "title": "Speed Tracking"
},
"Plot!diag_ekf_error": {
  "paths": [
    { "value": "/odometry/filtered.pose.pose.position.x", "label": "EKF x" },
    { "value": "/odom.pose.pose.position.x", "label": "ground truth x" }
  ],
  "title": "EKF vs Ground Truth (diagnostics)"
}

# 5. Hardware Section

Not applicable in the literal sense the task statement warns against — there is no PCB or FPGA in this project (hardware was dropped in favour of a sim-first platform; Jetson deployment remains future work). The equivalent real artifact for a software-only project — an editable interface design with a design explanation — is submitted above in place of it.

# 6. Individual Contribution

Rishi Satish: Defined the Pre-Drive screen's state/log panel bindings and the overall tab navigation structure in foxglove_layout.json.

Harshvardhan Singh: Built cone_viz.py (the classification-to-marker adapter powering Screen 2's 3D view) and the Drive View panel bindings.

Ayon Majumdar: Designed the Diagnostics screen (EKF-vs-ground-truth plot, lap counter) and produced the layout mockups in this document.

Note: this document is shared across all three submissions, per the "group members may upload the same shared file and must state their own contribution" instruction.